IoT Home Security and Intrusion Detection System

Table of contents

Key Features	2

What’s in the Box	3

System Requirements	5

Installation Guide	6

How to Use the System	9

Troubleshooting	11

Safety Information	14

Maintenance & Care	16

Technical Specifications	19

Warranty & Support	21

The Home Security Alarm System is designed to help protect your home or small business by detecting unwanted entry and alerting you immediately. It combines multiple sensors and an easy-to-use control system to give you peace of mind whether you are at home or away.

- This system includes a keypad control panel with an LCD display, along with door sensors, motion detection, and a built-in camera to monitor your space. If any unusual activity is detected, the system will trigger an alarm and notify you.

# Key Features

- Keypad Control Panel with LCD Screen
 Easily arm and disarm your system using the keypad. The LCD screen clearly displays system status, alerts, and current modes (e.g., armed, disarmed, triggered).

- Door/Window Magnetic Sensor
 Detects when doors or windows are opened unexpectedly.

- PIR Motion Sensor
 Uses infrared technology to detect movement inside a room.

- Built-in Camera
 Captures images or video when motion or intrusion is detected, allowing you to see what’s happening in real time.

- Instant Alerts & Alarm
 The system sounds an alarm and can notify you when a security breach occurs.

# What’s in the Box

Before setting up your system, please ensure all components are included in the package.

Included Items:

- ESP32 Control Unit with LCD Display
 The main controller that manages the entire alarm system and displays system status.

- Keypad Module
 Used to arm and disarm the system securely.

- PIR Motion Sensor
 Detects movement within the monitored area.

- Magnetic Door/Window Sensor
 Detects when a door or window is opened.

- Camera Module
 Captures images or video when motion or intrusion is detected.

- Buzzer (Alarm Siren)
 Produces a loud alert when the system is triggered.

- LED Indicator
 Provides visual feedback for system status (e.g., armed, triggered).

- Micro USB Cable
 Used to power the system.

# System Requirements

Before setting up your alarm system, ensure the following requirements are met for proper operation.

Power Requirements

- The system must be powered using the included Micro USB cable.

- It can be plugged into a standard wall adapter or connected to a household power supply.

- Ensure that a stable power source is available at all times for continuous protection.

Connectivity Requirements

- Wi-Fi connection is required for full system functionality.

  - Needed to connect the camera for live footage

  - Required to send alerts and store system activity logs

- The system uses built-in Wi-Fi and Bluetooth (via ESP32) for communication and setup.

Device Compatibility

- A smartphone, tablet, or computer is required to access the system dashboard.

- Compatible with:

  - iOS devices

  - Android devices

  - Any modern web browser (Chrome, Safari, Edge, etc.)

Web App Access

- The system is controlled and monitored through an online dashboard.

- Users must create an account to:

  - View live camera footage

  - Monitor alarm activity

  - Access system logs Access the dashboard here:
 https://iot-dashboard-5iyi19285-maxjegorovs-projects.vercel.app/

# Installation Guide

Follow the steps below to correctly set up your Home Security Alarm System.

Step 1: Choose Installation Locations

Before installing, decide where each component will be placed:

- Control Unit (ESP32 with LCD & Keypad)
 Place near the main entrance for easy access.

- Magnetic Door/Window Sensor
 Install on doors or windows that open to the outside.

- PIR Motion Sensor
 Mount in a corner of the room, facing the main area (avoid pointing at windows or heat sources).

- Camera Module
 Position where it has a clear view of the area you want to monitor.

- Buzzer & LED
 Place where they can be easily heard and seen.

Step 2: Install the Components

- Secure sensors and modules using screws, adhesive strips, or mounts.

- Ensure:

  - The magnetic sensor parts are aligned properly (magnet + switch).

  - The PIR sensor is elevated (around 2–2.5 meters high for best performance).

Step 3: Connect the System

- Connect all components to the ESP32 control unit as per your wiring setup.

- Double-check all connections before powering on.

Step 4: Power On the System

- Plug the system into a power source using the Micro USB cable.

- The LCD screen will turn on, displaying system status.

Step 5: Connect to Wi-Fi

- Use the keypad to begin setup (if applicable) or follow on-screen instructions.

- The system will connect to your Wi-Fi network using the built-in ESP32 module.

Make sure your Wi-Fi is stable and within range.

Step 6: Set Up Your Online Account

- Open a browser on your phone or computer.

- Go to:
 https://iot-dashboard-5iyi19285-maxjegorovs-projects.vercel.app/

- Create an account and log in.

From here you can:

- View live camera footage

- Monitor alarm activity

- Access system logs

Step 7: Test the System

- Arm the system using the keypad.

- Open a door or walk in front of the motion sensor.

- Check that:

  - The buzzer sounds

  - The LED activates

  - The camera captures activity

  - Alerts/logs appear on the website

Important Tips

- Do not install sensors near heat sources (radiators, direct sunlight).

- Ensure strong Wi-Fi signal near the control unit.

- Keep the control unit in a secure but accessible location.

# How to Use the System

Once installed, the alarm system can be easily controlled using the keypad and monitored through the online dashboard.

Arming the System

To activate the alarm system:

- Enter your security code on the keypad.

- The LCD screen will display that the system is armed.

- The LED indicator may light up to confirm activation.

- The Buzzer will chirp also to confirm activation.

When armed, all sensors (motion and door/window) are active.

Disarming the System

To deactivate the alarm system:

- Enter your security code on the keypad.

- The LCD screen will confirm that the system is disarmed.

- The LED indicator may light up to confirm deactivation.

- The Buzzer will chirp also to confirm deactivation.

The system must be disarmed to stop an active alarm.

When the Alarm is Triggered

The system will activate if:

- A door/window is opened (magnetic sensor)

- Movement is detected (PIR sensor)

When triggered:

- The buzzer will sound loudly

- The LED indicator will turn on

- The LCD screen will display an alert message

- The camera will capture footage

- The event will be recorded on the online dashboard

Viewing Camera and Activity Logs

You can monitor your system remotely:

- Open a web browser on your phone or computer

- Go to:
 https://iot-dashboard-5iyi19285-maxjegorovs-projects.vercel.app/

- Log into your account

From the dashboard, you can:

- View live camera footage

- Check previous alarm events

- Monitor system activity in real time

System Status Indicators

The system provides feedback using:

- LCD Screen → Displays current mode (armed/disarmed) and alerts

- LED Indicator → Shows system status visually

- Buzzer → Sounds during an alarm event

Daily Use Tips

- Always arm the system when leaving your home or premises

- Disarm immediately upon entry to avoid triggering the alarm

- Regularly check the dashboard for alerts or unusual activity

# Troubleshooting

If you experience issues while using the alarm system, refer to the common problems and solutions below.

Common Issues and Solutions

1. System does not power on

- Ensure the Micro USB cable is securely connected

- Check that the power source (wall adapter or supply) is working

- Try a different USB cable or power adapter

2. LCD screen is not displaying anything

- Confirm the system is powered on

- Check wiring connections to the LCD

- Restart the system by disconnecting and reconnecting power

3. Alarm does not trigger when movement is detected

- Ensure the system is armed

- Check that the PIR sensor is properly connected

- Make sure the sensor is not blocked

- Reposition the sensor (avoid placing too high/low or near heat sources)

4. Door/window sensor not working

- Check that the magnetic sensor parts are aligned correctly

- Ensure both parts are close together when the door/window is closed

- Inspect wiring connections

5. Buzzer does not sound during alarm

- Check buzzer connections to the control unit

- Ensure the system is properly triggered

- Test the buzzer separately if possible

6. Camera not showing live footage

- Ensure the system is connected to Wi-Fi

- Check that your internet connection is stable

- Restart the system

- Log out and back into the dashboard

7. Cannot connect to Wi-Fi

- Ensure correct Wi-Fi credentials are entered

- Check that the router is working

- Move the system closer to the router

- Restart both the router and the system

8. Website/dashboard not loading

- Check your internet connection

- Ensure you are using a supported web browser

- Try refreshing the page or using a different device

- Verify the correct website link is being used

9. False alarms (alarm triggers unexpectedly)

- Avoid placing the PIR sensor nearby:

  - Windows with direct sunlight

  - Heaters or radiators

  - Moving objects

- Adjust the position of the sensor

Resetting the System

If problems persist:

- Power off the system

- Wait 10–15 seconds

- Power it back on and restart setup if necessary

Support

If the issue cannot be resolved, please contact A00297703@student.tus.ie

# Safety Information

Please read and follow all safety instructions to ensure proper use of the alarm system and to avoid damage or injury.

Electrical Safety

- Only use the provided Micro USB cable and a suitable power adapter.

- Do not overload power outlets or use damaged cables.

- Ensure all wiring is secure before powering the system.

- Disconnect power before making any changes to the wiring or components.

Installation Safety

- Install all components securely to prevent them from falling or becoming loose.

- Do not place the system near water or in damp environments.

- Keep the system away from extreme heat sources (e.g., radiators, direct sunlight).

- Ensure sensors are mounted at appropriate heights and positions.

Operating Safety

- This system is designed as a security aid and should not be relied on as the only form of protection.

- Always double-check that the system is properly armed before leaving the premises.

- Do not attempt to modify or tamper with the internal components.

Camera & Privacy

- Use the camera responsibly and respect the privacy of others.

- Ensure the system is used in compliance with local privacy laws and regulations.

- Do not install cameras in private areas where recording is not permitted.

Maintenance Safety

- Power off the system before cleaning or performing maintenance.

- Do not use liquids or cleaning sprays directly on electronic components.

- Regularly check cables and components for signs of damage.

Important Notice

- This alarm system reduces risk but cannot guarantee complete protection against all intrusions or incidents.

- The manufacturer is not responsible for losses resulting from misuse or improper installation.

# Maintenance & Care

Regular maintenance will help ensure your alarm system operates reliably and remains in good condition over time.

Routine Checks

- Regularly check that all components are functioning correctly.

- Test the system by triggering the alarm (e.g., opening a door or walking past the motion sensor).

- Ensure that the LCD screen, buzzer, and LED indicator are working as expected.

Cleaning the System

- Keep all components clean and free from dust.

- Use a dry, soft cloth to clean the control unit, sensors, and camera.

- Do not use water, sprays, or harsh cleaning chemicals on any electronic parts.

Sensor Maintenance

- Ensure that the PIR motion sensor is not blocked by objects.

- Check that the magnetic door/window sensor remains properly aligned.

- Re-adjust sensor positions if performance decreases.

Camera Care

- Keep the camera lens clean for clear video quality.

- Avoid placing the camera in direct sunlight or areas with glare.

- Ensure a stable Wi-Fi connection for smooth video streaming.

Power and Connections

- Regularly inspect the Micro USB cable and connections for damage.

- Ensure the power supply remains stable and securely connected.

- Avoid bending or straining cables.

Software & System Monitoring

- Regularly log into the online dashboard to:

  - Check system status

  - Review alarm logs

  - Ensure the system is functioning correctly

Important Tip

- Perform a full system check at least once a month to ensure all components are working properly.

# Technical Specifications

The following specifications describe the main components and performance of the alarm system.

Control Unit

- Microcontroller: ESP32

- Display: LCD screen (for system status and alerts)

- Input: Keypad for user control

- Connectivity: Wi-Fi and Bluetooth (built-in)

- Power Supply: 5V via Micro USB

Sensors

PIR Motion Sensor

- Detects movement using infrared technology

- Detection Range: ~5–7 meters

- Field of View: ~120°

Magnetic Door/Window Sensor

- Detects opening and closing of doors/windows

- Consists of magnetic contact switch and magnet

Camera Module

- Captures images/video when motion is detected

- Supports live streaming via Wi-Fi

- Accessible through the online dashboard

Alert System

- Buzzer: audible alarm when triggered

- LED Indicator: Visual status indication (armed, disarmed, alarm triggered)

User Interface

- Keypad Input: Used for arming/disarming system

- LCD Display: Shows system status, alerts, and messages

Connectivity

- Wi-Fi: Required for remote monitoring and camera access

- Bluetooth: Used for local communication/setup (if applicable)

Power Requirements

- Input Voltage: 5V DC

- Powered via Micro USB cable connected to wall adapter or power supply

Operating Conditions

- Recommended Temperature: 0°C to 40°C

- Environment: Indoor use only

System Features

- Real-time intrusion detection

- Audible and visual alerts

- Remote monitoring via web dashboard

- Event logging and history tracking

# Warranty & Support

Thank you for choosing this Home Security Alarm System. We are committed to providing reliable and high-quality products.

Warranty Information

- This product is covered by a limited warranty of 12 months from the date of purchase.

- The warranty covers:

  - Manufacturing defects

  - Faulty components under normal use

What is Not Covered

The warranty does not cover:

- Damage caused by improper installation

- Misuse or neglect of the system

- Unauthorized modifications or repairs

- Damage caused by power surges or external factors

Support Services

If you experience any issues with your system, support is available to assist you.

You can:

- Refer to the Troubleshooting Section (Section 6) in this manual

- Access the online dashboard for system monitoring:
 https://iot-dashboard-5iyi19285-maxjegorovs-projects.vercel.app/

Contact Support

For further assistance please contact: A00297703@student.tus.ie

- Provide a description of the issue

- Include details such as system behavior and any error messages shown on the LCD screen

Returns & Replacements

- Products found to be defective within the warranty period may be repaired or replaced.

- Proof of purchase may be required for warranty claims.

Important Notice

This system is designed to improve security but does not guarantee complete protection against all incidents. Users are responsible for proper installation and use.